Cisco Passwords

- Purpose: Protect routers, switches, and firewalls from unauthorized access.

- Types:

- Console password: Direct access via console port.

- VTY password: Remote access via Telnet/SSH.

- Enable password: Access to privileged mode (not encrypted).

- Enable secret: Encrypted password (Type 5 MD5 hash), more secure.

- Encryption Types:

| Password Type | Purpose / Use | Encryption / Security Level |
| --- | --- | --- |
| Console password | Direct access via console port | Plain text / weak |
| VTY password | Remote access via Telnet / SSH | Plain text / weak |
| Enable password | Privileged mode access | Type 0 (plain text) or Type 7 (weak) |
| Enable secret | Privileged mode access (secure) | Type 5 (MD5), Type 8 (PBKDF2), Type 9 (scrypt) |

- Importance:

- Prevent unauthorized configuration

- Secure sensitive data

- Recommended: always use enable secret with a strong password